AppBox

Filesystem Layout

/ (rootfs)

/system (ext2, ro)

/system/config (tmpfs, populated from configuration partition)

/system/proc (procfs)

/system/dev (tmpfs)

/system/apps/<app_name>/<app_version> (not a mountpoint, scheme for system apps)

/data (ext3 or zfs)

/data/apps (not a mountpoint)

/data/apps/<app_name>/<app_version> (not a mountpoint, scheme for installed apps)

/data/<app_name> (not a mountpoint, directory for app to manage)

/apps (not a mountpoint, just a directory)

/apps/<app_name>/<app_version> (symlink to /data/apps/<something> or /system/apps/<something>)

/apps/<app_name>/default (symlink to default version of app)

Physical Disk Layout:

p1 (10MB): boot filesystem: kernel, initrd, extlinux: ext2 (not mounted)

p2 (2MB): config partition: header + tar

p3 (unknown): root filesystem: mounted on /system

p4 (unknown): data filesystem: mounted on /data

config partition:

First 1024 bytes reserved for:

[32 bytes]: Magic: SHA256("AppBox Configuration Partition"): {0x1d, 0xaa, 0x45, 0x9b, 0x97, 0x72, 0xef, 0x5d, 0x7d, 0x58, 0xc8, 0x14, 0x45, 0x82, 0x46, 0xc6, 0x9d, 0xbd, 0x1d, 0x53, 0xfa, 0x75, 0xf3, 0x96, 0x46, 0x29, 0x10, 0x25, 0x0b, 0xf7, 0xa4, 0xb6}

[1 byte]: Offset index

[7 bytes]: Reserved

[8 bytes]: Offset #0

[8 bytes]: Offset #1

[968 bytes]: Reserved

Tar contents (needed for early boot, may include other files)

boot/modules/*.ko

boot/modules/load

fstab

hostname

apps/<appname>

initrd:

/init

/tclkit

/modules/*.ko

/modules/load